# CMSC 411  Final Exam    F20          name______________________________

# userid ______________________

# Open book, open notes, calculators should be used, not shared.

Multiple choice, put   x   after a)  b)  c)   that is your answer.

Answer all questions with exactly one choice (zero, two or more is automatically wrong)

Pick closest answer if none is exact. Comments are ignored. Work very carefully!

After edit, save and   submit cs411 final final33a.doc

Q1:   A clock has a frequency time of  4.4 GHz, the period  is

  - 2.35 ps

  - 2.35 ns

  - 0.23 ns

  - 0.23 ps

Q2:   What is the average CPI, Clocks Per Instruction, for the following data table

| R format | 3 clocks | 10% |
| --- | --- | --- |
| J format | 4 clocks | 40% |
| M format | 4 clocks | 50% |

- 3.7

- 3.9

- 4.1

- 4.3

Q3:   What is the speedup for a specific program if 40,000 instructions are improved by a factor of 5,  30,000 instructions are improved by a factor of 5, and 10,000 instructions are not improved?

a)    about  2.5

- about  3.0

- about  3.5

- about  4.0

Q4:   In the twos complement system, using 4 bit words, the sum

11012 + 01112  is

a)    00012

- 00102

c)    01002

d)    10012

Q5:   In the sign magnitude system, using 8 bit dividend, 4 bit divisor,

the 4 bit result quotient  of  000011112  / 10112  is

a)   10112

b)   11012

c)   00112

d)   01012 									1  of  10

Q6:   Dividing a 64 bit number by a 40 bit number produces a quotient of

a)    16 bits

b)   24 bits

c)   32 bits

d)   64 bits

Q7:   A typical 32 bit two’s complement adder, e.g. project, has

a)   two 32 bit inputs and a 32 bit output

b)   two 32 bit inputs, a carry input, a carry output and a 32 bit output

c)   two 32 bit inputs, a carry input, and a 32 bit output

d)   one 32 bit input,  a carry input, a carry output, and a 32 bit output

Q8:   The general registers in the MIPS architecture (also used in the project) ignoring

control signals

- has two register addresses in and two data values out

- has three register addresses in, one data value in and three data values out

- has one 32-bit address in, one 32-bit value in, and one 32-bit value out

- has three register addresses in, one data value in and two data values out

Q9:   The memory address, MEM_addr, in the MIPS architecture (also used in the project)

- may be the memory address into the data memory

- may be the result of arithmetic instructions

- may be forwarded to the WB stage

- all of the above

Q10:   The MIPS architecture, also used in the project, uses

- a byte address for data

- a byte address for instructions

- a index for the cache

- all of the above

Q11:   In project part2a, in which stage are the multiplexors to forward for the beq instruction?

- ID

- IF

- EX

- None of the above

Q12:   A hazard is detected in part2b,  in stage

- ID

- IF

- EX

- MEM

2  of  10

Q13:   When stalling in the project part2b, registers are clocked in stages

- none

- IF and ID

- EX and MEM and WB

- MEM and WB

Q14:   Data forwarding in the project part2a for the  beq  instruction can forward

- WB_result and MEM_addr

b)   WB_result

c)   MEM_addr

d)   All of the above

Q15:   Assuming you implemented full data forwarding for the project, is a stall needed for the instruction sequence:

lw   $3,4($5)

lw   $5,4($4)

- yes

- no

Q16:   Assuming you implemented full data forwarding for the project, is data forwarding used for the instruction sequence:

cmpl  $4,$3

add   $4,$3,$3

- yes

- no

Q17:   Assuming you implemented full data forwarding for the project, is a stall needed for the instruction sequence:

lw    $5,3($4)

addi  $3,$5,100

- yes

- no

Q18:   Assuming you implemented full data forwarding for the project, will data forwarding be used by the instruction sequence

add   $5,$3,$4

sw    $7,6($5)

- yes

- no

Q19:   In part2a, a control signal on the ALU forwarding may include VHDL

- ex17: entity WORK.equal5 port map(EX_IR(20 downto 16), MEM_RD, BFMA1);

- ex19: entity WORK.equal5 port map(MEM_IR(31 downto 26), “100011”, BFMA3);

- a)  and  b)

- none of the above

3  of  10

Q20:   The hazard detection and data forwarding did not cause any action for a register number of zero  in a RD  (Register Destination) because

- by programmer convention, register zero always has a value zero

- register zero is fast enough to not cause a hazard or data forward

- by hardware design, register zero always has a value zero

Q21:   To get the best cache performance, assuming caches’s of the same size, the cache should be

- direct mapped

- two way set associative

- eight way set associative

- fully associative

Q22:   A four way set associative cache has how many compare, “=”, circuits for tag comparison

- 2

- 4

- 8

- none of the above

Q23:   A cache with 16 total words and four words per block has how many bits of the address in the index field ?

- 1

- 2

- 3

- 4

Q24:   With large block sizes,  in a small cache, cache miss percentage may increase.

- true

- false

Q25:   The write thru cache needs a “dirty bit”

- true

- false

Q26:   Two caches of the same size, with different number of words per block,

will typically have different number of capacity misses.

- true

- false

4  of  10

Q27:   What is the speedup, compared to a single processor, for a specific multiprocessor program that uses all processors on 9x108 instructions,  one half of the processors on 0.9x108 instructions and fully serial on 0.1x108 instructions,  with 60 processors in the multiprocessor.

- about  27

- about  29

- about  31

- about  33

Q28:  The addresses of instructions and data that you see in a debugger on a conventional computer like  LINUX.GL.UMBC.EDU, used in Homework 3, are

- physical addresses

- relocated addresses

- real addresses

- virtual addresses

Q29:  A virtual memory system uses 4KB pages. All addresses are byte addresses. The virtual address is 36 bits. The physical address is 32 bits. How many bits are in the virtual page number?

- 13

- 16

- 20

- 21

- 22

- 23

- 24

Q30:  A virtual memory system uses 8KB pages. All addresses are byte addresses. The virtual address is 35 bits. The physical address is 32 bits. How many bits are in the physical page number?

- 12

- 19

- 20

- 21

- 22

- 23

- 24

5  of  10

Floating point numbers have a fraction, F, and an exponent, E.

We will use decimal numbers for the fraction, F and exponent, E for this exam. (computers use binary numbers.)

A number has a value  V =  +/- F * 2^E .  e.g.  F=.5 and E=2 then V=2.0

We say a floating point number is normalized when  .5  F < 1.0 , thus the result

of a floating point computation must have F between .5 and 0.999.

Q31:   The decimal value, V, of the floating point number  -0.16x2-3  is

- -0.08

- -0.04

- -0.02

- -0.01

Q32:   The result of adding  .7x23  to  .9x22 is the normalized number

- .4x24

- 1.05x23

- .5x24

- .6x24

Q33:   The result of subtracting  .3x22   from   .7x23 is the normalized number

- .8x22

- .4x23

- .8x23

- .8x21

Q34:   The result of multiplying  .6x212  times  .5x212 is the normalized number

- .3x223

- .6x222

- .6x223

- .6x224

Q35:   The result of dividing  .6x29  by  .3x27 is the normalized number

- .5x25

- .5x24

- 1.0x24

- 2.0x23

6  of  10

Q36:   The empty cache shown below has a total of 16 words, organized as 4 blocks of 4 words per block.  The row numbers are not part of the cache. Given a MIPS 32 bit address with the fields shown below, how many bits are in the WORD SELECT field?

- 2

- 26

- 28

- 30

Address:

| TAG | INDEX | WORD SELECT | BYTE |
| --- | --- | --- | --- |

Cache:

| V | TAG | W0 | W1 | W2 | W3 | Row |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  | 0 |
| 0 |  |  |  |  |  | 1 |
| 0 |  |  |  |  |  | 2 |
| 0 |  |  |  |  |  | 3 |

Memory:

| ADDRESS |  | 68 | 72 | 76 | 80 | 84 | 88 | 92 | 96 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WORD CONTENTS |  | C | D | E | F | G | H | I | J | K |

Q37:   The contents of the memory word G at location 8410  =  10101002  is needed by the CPU. The cache block needs to be

a)

| V | TAG | W0 | W1 | W2 | W3 | Row |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  | 0 |
| 1 | 1 | F | G | H | I | 1 |
| 0 |  |  |  |  |  | 2 |
| 0 |  |  |  |  |  | 3 |

b)

| V | TAG | W0 | W1 | W2 | W3 | Row |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  | 0 |
| 0 |  |  |  |  |  | 1 |
| 1 | 1 | F | G | H | I | 2 |
| 0 |  |  |  |  |  | 3 |

c)

| V | TAG | W0 | W1 | W2 | W3 | Row |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  | 0 |
| 0 |  |  |  |  |  | 1 |
| 0 |  |  |  |  |  | 2 |
| 1 | 1 | F | G | H | I | 3 |

d)  none of the above								7  of  10

Q38:   In a computer with a simple CPU, L1 cache, L2 cache and RAM.

When an instruction is needed from the L1 cache, and there is a cache miss, then

- the L2 cache instructs the RAM to pass the instruction to the L1 cache

- the L2 cache must get the instruction and pass the instruction to the L1 cache

- the RAM passes the instruction to the L1 cache

- the L2 cache gets the instruction from RAM and passes the instruction to the CPU

Q39:   When data is to be stored in memory, and a write back cache is used, then

- the data is sent from the CPU to memory and is not placed in the cache

- the data is placed in the cache and then later sent to memory from the cache when the cache block is needed for different data.

- the data is sent from the CPU to both the cache and memory at about the same time

Q40:   Assume each cache miss causes a stall of 3 clocks in the pipeline, assume one instruction can be executed per clock, and ignore beginning and ending cases.

What is the speedup for a machine that has one miss per 7 instructions relative to a machine that has one miss per 4 instructions?

- about 1.15

- about 1.17

- about 1.2

- about 2.0

Q41:    In order to design a component to connect to a bus, you must know

- the number of connections to the bus

- the bus protocol

- the bus speed

- the bus logic

Q42:   To transfer a 400,000 byte file there is a 6 millisecond latency and a 20,000,000 bytes per second throughput. What is the total time to transfer the file?

- about   6 millisecond

- about  15 millisecond

- about  20 millisecond

- about  30 millisecond

Q43:  A program takes 60 seconds of CPU time and 50 seconds of I/O time. The machine gets a new CPU that improves CPU time by a factor of 2, what is the speedup?

- about  1.25

- about  1.39

- about  1.5

- about  1.71

8  of  10

Q44:   In VHDL we want to stall the PC and the instruction in the ID stage when the hazard detection unit sets  stall <= ‘1’.

The clock allowing the PC and ID stage to proceed is called ‘sclk’ and the system clock is called ‘clk’. The VHDL statement to compute ‘sclk’ for project part2b is

- sclk <= clk or not stall;

- sclk <= clk  or stall;

- sclk <= clk  and stall;

- sclk <= clk when not stall;

Q45:   In VHDL we want to shift the  beq  address two bits left in order to change a word address into a byte address called ‘byte_addr’,  a possible VHDL statement to do this, assuming the word address is named  ‘sign_ext’  is

- byte_addr <=  sign_ext(31 downto 2)  &   “00”;

- byte_addr <=  sign_ext(29 downto 0)  &    “00”;

- byte_addr <=  sign_ext(29 downto 0)  +   “00”;

- byte_addr <=  sign_ext(29 downto 2)  &   “00”;

Q46:   A multiprocessor is connected as an  n   dimensional array of processors.

Each processing element will have

- 4 connections

- 6 connections

- 8 connections

- n connections

Q47:   A multiprocessor is connected as an n-cube has 4096 processors. How many processors are directly connected to each processor?

- 10

- 11

- 12

- 2047

Q48:   What is the speedup, compared to a single processor, for a specific multiprocessor program that runs fully parallel for 60% of the time and fully serial the rest of the time with 10 processors.

- about 1.7

- about 1.8

- about 2.0

- about 2.2

9  of  10

Q49:   A disk rotates at 7200 RPM and has a published average seek time of 8 milliseconds and an overhead of 4 millisecond.  The transfer rate is 40,000,000 bytes per second. How long does it take to read a file of 400,000 bytes. Assume book correction factor on seek time.

- about  11 milliseconds

- about  14 milliseconds

- about  17 milliseconds

- about  20 milliseconds

- about  24 milliseconds

Q50:  A memory requires 1 cycle to get an address and word count from a bus. The memory access is 10 cycles to fetch four words. The next memory access begins immediately if more words are needed. The memory access overlaps the operation of putting data from the memory on a bus. The memory is four words wide, the bus is four words wide. The bus requires a bus cycle and two idle cycles at the end of each transaction and these are included in the transaction time. How many cycles does a transaction take to transfer 16 words. Do not include the time to get the address and word count.

- 39 or less

- 40

- 41

- 42

- 43

- 44

- 45

- 46

- 47

- 48 or more

End of exam.

Check that all questions have exactly one answer!

No project submittals or homework accepted after midnight December 12, 2020

Homework and Projects that have been graded will not be graded again.

All parties related to any copying get the UMBC Academic Misconduct

penalty on that project, homework, or exam. At least a zero for everyone involved.

Homework and Project scores will be posted on BlackBoard.

Please check and EMail any typo’s or missed submissions.

If any submissions not graded, submit another copy.

Do not use BlackBoard Drop Box.

Final letter grades will be posted when computed.

10  of  10