CMSC 411  Midterm Exam     F20                        name_________________________________

# open book, open note                                                       print your name

Multiple choice, type  x   after your choice a)  b)  c) …  Choose closest if not exact.

Answer all questions with exactly one choice (zero, two or more is automatically wrong)

After edit,  submit  cs411  midterm  midterm33b.doc

Q1: A 3.4GHz clock has a period of

a)   .33ns

- 3ns

- 30ps

- 300ps

Q2: What is the average CPI, Clocks Per Instruction, for the following data table

| G format | 2 clocks | 29% usage |
| --- | --- | --- |
| R format | 3 clocks | 11% usage |
| J format | 4 clocks | 40% usage |
| M format | 3 clocks | 20% usage |

- 3.3

- 3.4

- 3.6

- 4.0

Q3: What is the speed up, using Amdhal’s Law, of a machine where 40% of the

execution time is reduced by a factor of 3.

- 1.25

- 1.36

- 1.5

- 3

Q4: What is the MIPS rating of a computer with an average CPI of 4 and

a clock frequency of 1.6 GHz.

- 200

- 400

- 800

- 1600

Q5: Which VHDL statement produces a clock that is 200 MHz

- clk <= not clk after 2.5 ns;

- clk <= ‘0’ when clk’event and clk=’1’;

- clk <= not clk after 5 ns;

- clk <= clk + 5 ns;

# Q6: “open” is a reserved word in VHDL.

- true

- false

1  of  7

Q7. A barrel shifter can be built using multiplexors. The example shown in lecture could shift from zero places to 31 places using a five bit shift count. The least significant bit of the shift count controlled multiplexors that shifted the input

- zero or 1 place

- zero or 2 places

- zero or 8 places

- zero or 16 places

Q8: When the arithmetic unit does a divide, the arithmetic unit must know the location of the binary point in the divisor and the dividend.

- true

- false

- none of the above

Q9: A CPU with parallel multiply rather than serial multiply will need

- more area on the chip

- many more gates in the CPU

- less power consumption

- all of the above

Q10: A CPU is called a  64 bit CPU   because it has

- 64 bit floating point registers

- 64 bit registers for integers

- 64 bit data paths

- 64 bit memory bus

Q11: A good serial multiplier that multiplies 32 bits times 32 bits would normally require

- 128 gate times

- 16 clock times

- 64 clock times

- 1024 gate times

Q12: Multiplying 32 bits by 32 bits produces a product of

- 32 bits

- 33 bits

- 63 bits

- 64 bits

Q13: Dividing 64 bits by 32 bits produces a remainder of

- 32 bits

- 33 bits

- 63 bits

- 64 bits

2  of  7

Data for the next four floating point questions:

The value of floating point X is   0.1112 * 215    normalized

The value of floating point Y is   0.12 * 213    normalized

e.g. Y  has a fraction equal to 0.5  and an exponent of 2 equal to 13

A normalized floating point has a fraction  0.1111111112> f ≥ 0.12 and an exponent of base 2.

All operations normalize their results. All fractions are binary. All exponents are decimal.

Q14: What is the value of  0.112 * 23?

- 6.0

- 8.0

- 10.0

- 12.0

Q15: What is the normalized representation of  0.00012 ?

a)   fraction = 0.12    exponent = -1

b)   fraction = 0.12    exponent = -2

c)   fraction = 0.12    exponent = -3

d)   fraction = 1.02    exponent = -4

Q16: What is the sum of X plus Y, (X+Y) as a normalized floating point number?

- fraction =  1.02       exponent = 15

- fraction =  0.12       exponent = 15

- fraction =  0.12       exponent = 16

- fraction =  0.12       exponent = 17

Q17: What is the difference of X minus Y, (X-Y), as a normalized floating point number?

- fraction =  0.12       exponent = 15

- fraction =  0.1102   exponent = 14

c)   fraction =  0.1102   exponent = 15

d)   fraction =  0.1102   exponent = 16

Q18: What is the product of X times Y, (X*Y), as a normalized floating point number?

- fraction =  0.1112    exponent = 29

- fraction =  0.12        exponent = 29

- fraction =  0.1112    exponent = 27

- fraction =  0.1112    exponent = 28

Q19: What is the value of X divided by Y, (X/Y) as a normalized floating point number?

- fraction =   0.12       exponent = 1

- fraction =   0.1112   exponent = 1

- fraction =   0.1112   exponent = 2

- fraction =   0.1112   exponent = 3

3  of  7

Q20: Machine X has a 0.266GHz clock and

type                   CPI                       count

| R format | 2 | 250 instructions |
| --- | --- | --- |
| M format | 3 | 500 instructions |

What is the MIPS, Million Instructions Per Second, for machine X

- 100      answers are approximate, choose closest

- 125

- 133

- 180

Q21: Machine Y has a 2GHz clock and

type                   CPI                       count

| R format | 1 | 800 instructions |
| --- | --- | --- |
| M format | 2 | 1200 instructions |

Approximately how many micro-seconds does it take Machine Y to execute 10000 instructions

- 5

- 8

- 13

d)  36

Q22: We want to speed up machine X.  Which will give improvement to machine X:

- decrease the number of clocks to do a R format instruction to 1 clocks

- reduce the number of clocks to do a M format instruction to 2 clocks

- increase the clock speed

- all of the above

Q23: In digital circuitry, storage elements are usually clocked.

Between clocked storage elements is typically found:

- combinational logic

- flip flops

- registers

- just wires

Q24: What is the decimal value of this twos complement four-bit number   1011

- -2

- -3

- -4

- -5

- -6

Q25: When benchmarking some code that takes very little time, use

the benchmarking technique:

- long measurement time

- double difference method

- repeat the benchmark many times

- run more than 24 hours

4  of  7

Enter answers on next page.

Circuits, VHDL statements and truth tables are for the questions below.

Circuit A                         Circuit B                                      Circuit C

VHDL 1        d <= not (a or b) and c;

VHDL 2        d <= a when c=’1’ else b;

VHDL 3        d <=  (not a and b) or not c;

truth tables: inputs a, b, c and output d

Ttable X                                        Ttable Y                               Ttable Z

a b c | d            a b c | d          a b c | d

------+--            ------+--          ------+--

0 0 0 | 0            0 0 0 | 1	     0 0 0 | 0

0 0 1 | 0            0 0 1 | 0          0 0 1 | 1

0 1 0 | 1            0 1 0 | 1          0 1 0 | 0

0 1 1 | 0            0 1 1 | 1          0 1 1 | 0

1 0 0 | 0            1 0 0 | 1          1 0 0 | 0

1 0 1 | 1            1 0 1 | 0          1 0 1 | 0

1 1 0 | 1            1 1 0 | 1          1 1 0 | 0

1 1 1 | 1            1 1 1 | 0          1 1 1 | 0

5 of 7

Q26: Circuit C can be represented by  VHDL statement?

- VHDL 1

- VHDL 2

- VHDL 3

Q27: Circuit B can be represented by  VHDL statement?

- VHDL 1

- VHDL 2

- VHDL 3

# Q28: Circuit A is represented by truth table, Ttable?

- Ttable X

- Ttable Y

- Ttable Z

Q29:  Circuit B is represented by truth table, Ttable?

- Ttable X

- Ttable Y

- Ttable Z

Q30: VHDL statement VHDL 2  is represented by truth table, Ttable?

- Ttable X

- Ttable Y

- Ttable Z

Q31: Use Amdhal’s law to determine the speedup when the time for 300 instructions is reduced by a factor of 4, the time for 250 instructions is reduced by a factor of  2  and

the remaining 100 instructions are unchanged.

a)   1.17

b)   1.5

c)   2.17

d)   2.7

Q32: The one’s complement sum of  the ones’s complement numbers

1010112   +  0011112    is

- 1000112

- 1110102

- 0001012

- 1111002

Q33: The unsigned product of the unsigned numbers   0102 * 0112  is

- 01102

- 0110002

- 0011002

- 0001102

6 of 7

Q34: The ones complement difference of  the ones complement numbers 1010102 - 0001112  is

a)  0101102

b)  1000112

c)  1100012

d)  1101102

Q35: A subtraction, X-Y, is performed in twos complement using a hardware adder using

- X added to the ones complement of Y

- X subtracted from the ones complement of Y with carry in set to 1

- One’s complement of Y added to X with carry in set to 1

- None of the above

Q36: A microprogram controller is typically characterized by

- having a branch instruction in every instruction

- instruction length that is economized to a few bits

- executing an instruction every clock

- all of the above

Q37: Comparing two computers A and B, from different manufacturers that sell for about the same price

- all benchmarks should run close to the same time on both computers

- all benchmarks should run faster on either computer A or computer B

- there will be some benchmark that runs faster on computer A and a different benchmark

that runs faster on computer B

Q38: When attempting to determine which computer has a faster CPU for you

- do not let the benchmark start before midnight and end after midnight

- try to have only the benchmark program running

- run benchmarks similar to programs you use

- all of the above

Q39: When comparing two computers, if one computer runs faster on all the SPEC integer benchmarks then it will run faster on all the SPEC floating point benchmarks

- true

- false

Q40: On the same computer, running a set of benchmarks will give the same results for various operating systems such as Microsoft Windows and Linux

  - true

  - false

Q41: It is possible for a hyperthread processor to take more time running two programs simultaneously, in one core, than the time to run the two programs sequentially.

- true

- false

- none of the above

end of exam

7 of 7